Firefly Breath — Technical Build Guide

Parts, wiring, firmware, and app-side code for the breath-controlled recorder game

Technical Companion to the Firefly Breath Pitch  |  Prepared for Saurus

# 1. System Overview

The system has three layers. Data flows one way, left to right, on every blow:

| Layer | What it does | What it's made of |
| --- | --- | --- |
| 1. Sensing | Turns breath pressure into an electrical signal | Recorder flute + air-pressure sensor |
| 2. Firmware | Reads that signal, cleans it up, decides which "band" (soft/medium/firm) it falls in, sends a simple message | Arduino or ESP32 board running C++ code |
| 3. Game / App | Receives that message, plays the matching note, and animates the firefly's hop | A laptop, tablet, or phone running a web page (HTML/JS) |

A term used throughout this document: "microcontroller" just means a small, cheap computer chip (like the one on an Arduino board) that can read sensors and send data, but has no screen of its own — it needs to be connected to something else (in this case, a laptop or tablet) to show the game.

# 2. Bill of Materials

## 2.1 Core sensing hardware (required)

| Part | Approx. price | Why this one |
| --- | --- | --- |
| Recorder-type flute (already have) | — | Your existing controller body |
| MPXV7002DP differential pressure sensor breakout | $15–19 | This is the same part used in real DIY MIDI "breath controller" projects — it reads a wide, gentle pressure range (about ±2 kPa) which matches human blowing, not industrial airflow. Widely documented, so troubleshooting help is easy to find. |
| Silicone/Tygon tubing (1/8" ID) + a barbed fitting | $3–6 | Connects the flute's mouthpiece airway to the sensor's intake port without leaking. |
| Arduino Uno (or Arduino Nano) | $8–25 (clone vs. official) | Reads the sensor's analog voltage output and runs the firmware in Section 4. |
| Breadboard + jumper wires | $5–8 | For prototyping the wiring before soldering a permanent version. |
| 3× 0.1µF ceramic capacitors (optional) | $1 | Sensor manufacturers recommend these to filter electrical noise on the sensor's power line — improves reading stability. |
| USB cable (board to computer) | $2–5 | Powers the Arduino and carries data to the game in the tethered/prototype setup (Section 3). |

## 2.2 If you want a wireless / standalone version later

| Part | Approx. price | Why |
| --- | --- | --- |
| ESP32 dev board (instead of Arduino Uno) | $5–10 | Same programming environment as Arduino (same code language, Arduino IDE supports it directly), but with Wi-Fi and Bluetooth built in — no separate wireless module needed. Lets the sensor unit talk to a tablet/screen without a USB cable. |
| Small LiPo battery + charging module | $8–12 | Makes the flute unit portable instead of tethered to a wall outlet. |
| Small speaker + amplifier module (optional) | $5–10 | Only needed if you want the flute unit itself to make sound, instead of relying on the connected screen's speakers. |

Recommendation: build the tethered Arduino Uno version first (Section 3). It's simpler to debug because you can watch the raw sensor numbers on your computer while you calibrate the pressure bands. Move to ESP32 + wireless only once the pressure bands and game feel right.

# 3. Wiring

The MPXV7002DP breakout has 4 pins: VCC (power), GND (ground), and two analog output pins (only one is needed for a simple gauge-pressure reading).

- Sensor VCC → Arduino 5V

- Sensor GND → Arduino GND

- Sensor analog output pin → Arduino analog input pin A0

- Leave the sensor's second pressure port open to open air (this makes it measure your breath pressure relative to the room, which is what you want)

- Tubing runs from the flute's mouthpiece/windway to the sensor's intake port

If you added the optional noise-filtering capacitors: one goes between VCC and GND right at the sensor, as close to the sensor's pins as possible — this is standard practice recommended by the sensor's datasheet and by hobbyists who've built similar breath controllers.

# 4. Firmware (Arduino code)

This runs directly on the Arduino board. In plain terms, it repeats a loop many times per second: read the sensor, smooth out the noise, figure out which pressure "band" the child is blowing at, and if it's a fresh blow (not still the same one from a moment ago), send a short message like BLOW,MEDIUM over the USB cable to the computer.

## 4.1 Key ideas explained

- Analog read: analogRead() gets a number from 0–1023 representing the voltage the sensor is currently outputting — higher pressure means a higher (or lower, depending on wiring) number.

- Smoothing (moving average): instead of trusting one single reading (which can jitter), the code keeps the last 8 readings and averages them. This is a moving average — a standard way to remove noise from a live sensor signal.

- Calibration: at the start of a session, the code asks for a 3-second "soft" blow and a 3-second "firm" blow, and remembers the lowest and highest smoothed values it saw. Every later reading is judged as a percentage of that personal range, so the game is fair whether a small 5-year-old or a bigger 10-year-old is playing.

- Debounce: after sending a BLOW message, the code waits half a second before it's allowed to send another one, so a single long blow doesn't get counted as dozens of rapid jumps.

## 4.2 Full firmware code

// FireflyBreath — Arduino firmware // Reads the MPXV7002DP pressure sensor, smooths it, calibrates it // per child, and sends BLOW,<band> messages over Serial (USB). const int SENSOR_PIN = A0; const int SAMPLE_COUNT = 8; // how many readings to average int samples[SAMPLE_COUNT]; int sampleIndex = 0; int minPressure = 1023; // set during calibration (softest blow) int maxPressure = 0; // set during calibration (firmest blow) unsigned long lastTriggerTime = 0; const unsigned long DEBOUNCE_MS = 500; // cooldown between triggers void setup() { Serial.begin(9600); for (int i = 0; i < SAMPLE_COUNT; i++) samples[i] = 0; } int readSmoothedPressure() { samples[sampleIndex] = analogRead(SENSOR_PIN); sampleIndex = (sampleIndex + 1) % SAMPLE_COUNT; long sum = 0; for (int i = 0; i < SAMPLE_COUNT; i++) sum += samples[i]; return sum / SAMPLE_COUNT; } void calibrate() { Serial.println("CALIBRATE_SOFT"); // app shows "give a soft blow" unsigned long start = millis(); while (millis() - start < 3000) { int val = readSmoothedPressure(); if (val < minPressure) minPressure = val; delay(20); } Serial.println("CALIBRATE_FIRM"); // app shows "give a firm blow" start = millis(); while (millis() - start < 3000) { int val = readSmoothedPressure(); if (val > maxPressure) maxPressure = val; delay(20); } Serial.print("CALIBRATED,"); Serial.print(minPressure); Serial.print(","); Serial.println(maxPressure); } String bandFor(int val) { int range = maxPressure - minPressure; if (range <= 0) return "NONE"; float pct = (float)(val - minPressure) / range; if (pct < 0.15) return "NONE"; // not blowing / below threshold if (pct < 0.45) return "SOFT"; if (pct < 0.75) return "MEDIUM"; return "FIRM"; } void loop() { // Computer sends a single 'C' character to start calibration if (Serial.available() && Serial.read() == 'C') { calibrate(); } int val = readSmoothedPressure(); String band = bandFor(val); if (band != "NONE" && millis() - lastTriggerTime > DEBOUNCE_MS) { Serial.print("BLOW,"); Serial.println(band); lastTriggerTime = millis(); } delay(15); // loop roughly 65 times per second }

# 5. Game / App Layer

This is the part the child actually looks at: the firefly, the lily pads, and the sound. It runs as a simple web page (HTML + JavaScript), which has two advantages for a first prototype: it needs no app-store install, and modern browsers can talk directly to an Arduino over USB using something called the Web Serial API (a browser feature, currently supported in Chrome and Edge, that lets a web page read data from a USB device like your Arduino without any extra software).

## 5.1 Key ideas explained

- Web Serial API: lets the web page "listen" to the Arduino's USB connection directly, so the moment the firmware sends BLOW,MEDIUM, the web page receives it almost instantly.

- Web Audio API: a built-in browser tool for generating real sound (not pre-recorded audio files) — this is what plays an actual musical note (a specific frequency, e.g. 523.25 Hz = the note C5) the instant a blow is detected.

- Oscillator: the Web Audio API component that generates a single musical tone at a chosen pitch — this is the digital equivalent of a synthesizer's note generator.

## 5.2 Example app code

// firefly-breath.js — runs in the browser // 1. Connect to the Arduino over USB let port, reader; async function connectToArduino() { port = await navigator.serial.requestPort(); // user picks the device await port.open({ baudRate: 9600 }); const decoder = new TextDecoderStream(); port.readable.pipeTo(decoder.writable); reader = decoder.readable.getReader(); readLoop(); } // 2. Continuously read incoming messages from the Arduino async function readLoop() { let buffer = ""; while (true) { const { value, done } = await reader.read(); if (done) break; buffer += value; let lines = buffer.split("\n"); buffer = lines.pop(); // keep any incomplete line for next time for (const line of lines) handleLine(line.trim()); } } // 3. React to each message function handleLine(line) { if (line.startsWith("BLOW,")) { const band = line.split(",")[1]; // "SOFT" ¦ "MEDIUM" ¦ "FIRM" playNote(band); jumpFirefly(band); } } // 4. Play a real musical note for that pressure band const noteFrequency = { SOFT: 392.00, MEDIUM: 523.25, FIRM: 659.25 }; // G4, C5, E5 const audioCtx = new (window.AudioContext ¦¦ window.webkitAudioContext)(); function playNote(band) { const osc = audioCtx.createOscillator(); const gain = audioCtx.createGain(); osc.frequency.value = noteFrequency[band] ¦¦ 440; osc.type = "sine"; // soft, flute-like tone gain.gain.setValueAtTime(0.3, audioCtx.currentTime); gain.gain.exponentialRampToValueAtTime(0.001, audioCtx.currentTime + 0.4); osc.connect(gain).connect(audioCtx.destination); osc.start(); osc.stop(audioCtx.currentTime + 0.4); } // 5. Move the firefly sprite on screen function jumpFirefly(band) { const hopSize = { SOFT: 40, MEDIUM: 80, FIRM: 140 }[band] ¦¦ 40; // pixels // ...update the firefly sprite's position/animation by hopSize // and light up / advance to the next lily pad }

This same handleLine() function is where the pursed-lip "precision hold" level and the rhythm-sync level plug in later — they just watch the incoming BLOW messages over a longer window instead of reacting to a single one (e.g. "was the band MEDIUM for the last 2 seconds without dropping to NONE?").

# 6. Communication Protocol (Arduino ↔ App)

Keeping the message format this simple (plain text lines, comma-separated) means either side can be tested independently — you can literally open the Arduino IDE's Serial Monitor and read the same messages the web app receives, which makes debugging far easier for a first build.

| Message | Meaning | Sent by |
| --- | --- | --- |
| C | "Start calibration" | App → Arduino |
| CALIBRATE_SOFT | "Please give a soft/gentle blow now" | Arduino → App |
| CALIBRATE_FIRM | "Please give a firm/strong blow now" | Arduino → App |
| CALIBRATED,<min>,<max> | Calibration finished; here are this child's pressure range values | Arduino → App |
| BLOW,<band> | A qualifying blow happened; band is SOFT, MEDIUM, or FIRM | Arduino → App |

# 7. Testing & Calibration Protocol

- Bench-test the sensor alone first: upload only a simple sketch that prints raw analogRead() values to the Serial Monitor while you blow gently, then firmly — confirm the numbers rise and fall smoothly before adding any game logic.

- Test smoothing: compare the raw values to the smoothed values side by side — the smoothed line should look like a gentle curve instead of a spiky one.

- Test with real children across the target age range (5 and 10 year olds, or the closest available stand-ins) before finalizing the SOFT/MEDIUM/FIRM percentage cut-offs (currently 15/45/75% in the sample code) — these numbers are a starting point, not a final answer.

- Test debounce timing during actual play — 500ms is a starting value; if kids' single blows are still triggering double-jumps, increase it; if it feels laggy, decrease it.

- Airtightness check: make sure the tubing-to-mouthpiece connection doesn't leak air, or pressure readings will be artificially low and inconsistent.

# 8. Suggested Build Phases

- Phase 1 — Sensor bring-up: wire the MPXV7002DP to the Arduino, confirm raw readings on the Serial Monitor.

- Phase 2 — Firmware: add smoothing, calibration, band logic, and BLOW messages (Section 4).

- Phase 3 — Minimal web app: connect via Web Serial API, log incoming BLOW messages to the browser console — no visuals yet, just confirm the data pipeline works end-to-end.

- Phase 4 — Sound: add the Web Audio API note-playing (Section 5.2, step 4) so each blow produces a tone.

- Phase 5 — Visuals: add the firefly sprite, lily pads, and hop animation (Section 5.2, step 5).

- Phase 6 — Levels: layer in the pursed-lip "precision hold" and rhythmic "sync to the beat" mechanics from the pitch document, each watching the BLOW stream over a longer time window.

- Phase 7 (optional) — Go wireless: swap the Arduino Uno for an ESP32, replace the USB/Web-Serial link with Bluetooth Low Energy (BLE), and move the game to a tablet.

# 9. Sources & References

Synth Hacker — DIY MIDI Breath Controller (build log using a differential pressure sensor + Arduino) — http://synthhacker.blogspot.com/2019/09/diy-midi-breath-controller.html

little-scale — Breath sensor made from an MPXV7002 (real-world breath-instrument sensor build) — http://little-scale.blogspot.com/

Hackster.io — Digital Manometer/CPAP Machine Monitor using MPXV7002DP with Arduino — https://www.hackster.io/binsun148/digital-manometer-cpap-machine-monitor-c1620a

The Answer is 42 — Using the MPXV7002DP Differential Pressure Sensor with an Arduino Uno — http://langster1980.blogspot.com/2017/04/using-mpx7002dp-differential-pressure.html

Hackaday.io — USB MIDI Breath Controller (pressure sensor + Arduino + capacitor noise filtering) — https://hackaday.io/project/161678-usb-midi-breath-controller

MIT Emergency Ventilator project — Pressure Measuring with MPXV7002DP and Arduino's analog input — https://emergency-vent.mit.edu/controls/pressure-measuring/

Cirkit Designer — MPXV7002DP Pressure Sensor pinouts, specs, and examples — https://docs.cirkitdesigner.com/component/f397776b-a86f-e94f-d7e6-4e90ac3152fc/mpxv7002dp-pressure-sensor

Note: the sample code in this document is a working starting point, not production-tested firmware — test each phase (Section 8) on the bench before relying on it in front of a child.